Senvia 3D Asset System

Solution C: code library + GLB/JSON asset database + live viewer

Technical design for the Sensovibe software team. Framework-neutral: the core is TypeScript + Three.js and plugs into React, Angular, Vue, plain web or a mobile WebView.

# 1. Goal and principles

Turn the 3D equipment library into a permanent part of Senvia: every customer asset is shown as a correct 3D machine, its measurement points are linked to real sensors, and live condition colours, rotation and See-inside views come from Senvia data.

- One source of truth: the model library (code) generates every 3D model; nothing is hand-edited in the app.

- Fast at run time: the app loads small pre-built files (GLB + JSON), not the whole library.

- Behaviour travels with the model: rotation speeds, points, bearings, flow and See-inside rules are stored in JSON beside each GLB.

- Customer data stays in Senvia: asset IDs, sensor channels and readings never leave your system.

- Versioned and testable: every model has a version; automated checks (the bug-hunt script) run before any model is published.

# 2. Architecture

| Layer | What it is | Runs where | Owner |
| --- | --- | --- | --- |
| 1 Model library | TypeScript package with all machine and component builders, helpers, materials, register (IDs, colours, points) | Developer PC, CI server | 3D / R&D team |
| 2 Build and QA pipeline | CLI that builds every model, runs the checks, exports GLB + JSON, compresses, uploads | CI server (headless Chromium) | DevOps |
| 3 Asset store | Versioned GLB + JSON files per model, plus thumbnails | Object storage (S3 / MinIO / Azure Blob) + CDN | DevOps |
| 4 Senvia database | Customer assets linked to a model ID; points linked to sensors; readings and alarms | Senvia backend (existing DB) | Backend team |
| 5 Viewer component | Small runtime that loads GLB + JSON and applies rotation, See-inside, flow, sensors, point colours from live data | Browser, mobile WebView | Frontend team |
| 6 Live data link | WebSocket/MQTT feed of point status and speed into the viewer | Senvia backend → client | Backend + frontend |

Data flow: library → build pipeline → asset store (GLB + JSON) → viewer loads model by model_id → Senvia API supplies the customer asset, point-to-sensor mapping and live values → viewer colours points, spins shafts at running speed, shows alarms.

# 3. Repository structure (monorepo)

senvia-3d/

packages/

core/              # model library (from today's index.html)

src/materials.ts       paint(), PAL (RAL colours), metal materials

src/geometry.ts        xcyl, lathe, extrudeX, gear, scroll, belt ...

src/parts/             motor, generator, coupling, pulley, plummer,

impeller, fanWheel, planetary, seal, bearings

src/machines/          one file per machine: es.ts, mill.ts, kiln.ts ...

src/components/        one file per component: dgbb.ts, gcpl.ts ...

src/rules/             connectShafts, autoBearings, mergeStatic,

sensors, flow, designRotation

src/register.ts        REG ids, names, industries, drives, points

src/index.ts           buildModel(id, options) -> THREE.Group + meta

export/            # CLI: build, check, export

src/cli.ts             senvia3d build --all | --id EQ-UTL-001

src/checks/            bug-hunt checks (A-G) as automated tests

src/gltf.ts            GLTFExporter + Draco/meshopt compression

src/meta.ts            writes model JSON (schema in section 5)

viewer/            # runtime for the Senvia app (no builders inside)

src/Viewer.ts          load, render loop, controls

src/behaviours/        rotation, seeInside, flow, sensors, arrows,

reverseWarning, pointMarkers

src/api.ts             public methods and events (section 6)

viewer-react/ | viewer-angular/   thin wrappers (optional)

apps/

playground/        # today's library page, for the 3D team

assets/              # build output (not committed): /models/<id>/<ver>/

Migration from today: the published library is one HTML file. Step 1 is to split it into the files above without changing behaviour, then prove it with the same bug-hunt script (all 94 models must give the same results).

# 4. GLB file conventions

| Item | Rule |
| --- | --- |
| Units and axes | 1 unit = 1 m; shaft axis along X, Y up; origin on the baseplate centre-line |
| Measurement points | Empty nodes named MP_01_MOTOR_NDE_BEARING … in ISO order (driver NDE = 01) |
| Rotating parts | Groups named ROT_nn; spin axis = node local X (or Y for vertical); speed ratio and direction in JSON |
| Internals | Nodes under INTERNAL_ root; hidden by default, shown in See-inside mode |
| Guards, flow, sensors | GUARD_nn, FLOW_nn, SENSOR_MP_nn nodes; hidden by default |
| Materials | One per paint colour, named PAINT_RAL_5014_…; metals STEEL, CAST, BRONZE, COPPER; See-inside uses a transparent copy made in the viewer |
| Compression | Draco or meshopt via gltf-transform; target ≤ 3 MB per machine, ≤ 1 MB per component |
| Performance | ≤ 250 draw calls, ≤ 60k triangles with every feature on (already met: max 205 / 59.7k) |

# 5. Model JSON schema (one per GLB)

The JSON carries everything the GLB cannot: speeds, directions, bearing data, point metadata, flow and rules. Example (shortened):

{

"model_id": "EQ-UTL-001", "version": "1.4.0", "name": "End-suction pump",

"industry": "Utilities & process", "drive": "Direct coupled",

"glb": "models/EQ-UTL-001/1.4.0/model.glb", "units": "m",

"design_rotation": "CW_FROM_DE", "direction_sensitive": true,

"paint": [{"part": "Motor", "ral": "RAL 5014", "hex": "#637D96"},

{"part": "Pump",  "ral": "RAL 3000", "hex": "#A72920"}],

"rotating": [

{"node": "ROT_01", "role": "motor_shaft", "axis": "x", "ratio": 1.0},

{"node": "ROT_05", "role": "impeller",    "axis": "x", "ratio": 1.0,

"blades": 6, "blade_pass_order": 6}],

"points": [

{"id": "MP_01", "label": "Motor NDE bearing", "node": "MP_01_MOTOR_NDE_BEARING",

"directions": ["H","V","A"], "bearing": "B1", "shaft_ratio": 1.0},

{"id": "MP_03", "label": "Pump DE bearing", "directions": ["H","V","A"],

"bearing": "B3", "shaft_ratio": 1.0}],

"bearings": [

{"id": "B3", "type": "rolling", "bore_mm": 100, "od_mm": 180,

"width_mm": 45, "designation": null,

"fault_orders": {"BPFO": null, "BPFI": null, "BSF": null, "FTF": null}}],

"flow": [{"type": "liquid", "node": "FLOW_01"}],

"internals": ["INTERNAL_IMPELLER", "INTERNAL_SEAL", "INTERNAL_BEARINGS"],

"stats": {"draw_calls": 38, "triangles": 21450, "size_kb": 2155},

"checks": {"passed": true, "run": "2026-09-28", "by_design": []}

}

- designation and fault_orders are filled per customer asset (real bearing numbers), not in the generic model.

- ratio is the speed of that part relative to the driver: the viewer multiplies it by the live running speed.

# 6. Viewer API

| Method / event | Purpose |
| --- | --- |
| load(modelId, version?) | Fetch JSON + GLB from the asset store (cached) |
| bindAsset(asset) | Apply customer asset data: name, bearing designations, paint overrides, point-to-sensor map |
| setRunning(rpm ¦ null) | Spin all ROT nodes at rpm × ratio (null = stopped) |
| setDirection("CW_FROM_DE"¦"CCW_FROM_DE") | Reverse; shows warning on direction-sensitive models |
| setSeeInside(bool) / setFlow(bool) / setSensors(bool) / setGuards(bool) | Mode switches (same as today) |
| setPointStatus(mpId, {level, value, unit, time}) | Colour point by severity (normal / alert / danger) and show value on tap |
| focusPoint(mpId) | Fly camera to a point |
| highlightPart(node) | Glow a part (e.g. bearing with BPFO alarm) |
| screenshot(): Blob | PNG for reports |
| on("pointClick", cb) | Open trend / spectrum for that point in Senvia |
| on("ready" ¦ "error", cb) | Load lifecycle |

const v = new SenviaViewer(containerEl, { assetBase: "https://cdn.sensovibe.in/3d" });

await v.load("EQ-UTL-001");

v.bindAsset(asset);                       // from Senvia API

v.setRunning(asset.live.rpm);

socket.on("point-status", s => v.setPointStatus(s.mp, s));

v.on("pointClick", mp => router.open("/trend/" + asset.id + "/" + mp));

# 7. Database tables (Senvia backend)

| Table | Key columns |
| --- | --- |
| model_catalog | model_id, name, industry, drive, latest_version, status (built / to build), register data |
| model_version | model_id, version, glb_url, json_url, size_kb, checks_passed, created_at |
| asset | asset_id, customer_id, site_id, tag_no, model_id, model_version (pinned), kW, rpm_nominal, paint_override |
| asset_point | asset_id, mp_id, sensor_id, channel, direction (H/V/A/X/Y), mounting (stud/magnet), enabled |
| asset_bearing | asset_id, bearing_ref (B1…), designation (e.g. 6316 C3), maker, fault_orders JSON |
| asset_photo | asset_id, point_id (optional), url, caption, taken_at (camera photos per asset/point) |
| point_status | asset_id, mp_id, level, overall_value, unit, standard (ISO 20816 zone), updated_at |
| alarm | asset_id, mp_id, fault_type (BPFO, unbalance, misalignment…), severity, opened_at, closed_at |

Severity colours follow ISO 20816 zones by default (A/B green, C amber, D red) and can be overridden per asset. Fault types from Senvia analytics map to parts in the JSON (e.g. BPFO on B3 highlights bearing B3).

# 8. Linking models to real sensors

- Engineer creates an asset in Senvia and picks a model (search by name or ID).

- Senvia shows the model with its MP points; the engineer taps each point and selects the sensor and channel (8–32 channel IEPE system or wireless node).

- Bearing designations are entered once per asset; Senvia computes the fault frequencies and stores them in asset_bearing.

- Optional: camera photos of the real machine are attached to the asset or to a point, and shown beside the 3D view.

- From then on, live readings colour the points, and alarms highlight the affected part in 3D.

# 9. Build, test and release pipeline

| Step | What happens |
| --- | --- |
| 1 Commit | 3D team changes a builder or adds a machine in packages/core |
| 2 Build | CI builds every changed model headless |
| 3 Check | Bug-hunt checks run as tests: shaft continuity, bearings on journals and ISO ratios, vane and volute direction, floating parts, point order, budget. Any failure blocks release |
| 4 Export | GLB (compressed) + JSON + thumbnail per model; version bumped (major = geometry/points change, minor = visual, patch = fix) |
| 5 Publish | Upload to asset store under /models/<id>/<version>/; update model_catalog and model_version |
| 6 Adopt | Existing customer assets stay pinned to their version until an engineer upgrades them (points never renumber silently) |

# 10. Mobile and performance

- Web: the viewer is a plain ES module; wrappers for React/Angular are 50–100 lines each.

- Mobile: embed the web viewer in a WebView (Flutter, React Native, native) and pass data through a message bridge; one code base for all.

- Load only one model at a time; cache GLB/JSON on the device; shadows and See-inside materials are created on demand.

- Budgets already met by today's library: ≤ 205 draw calls and ≤ 59.7k triangles per machine with every feature on.

# 11. Security and ownership

- Asset store behind signed URLs; generic models are not customer data, but asset bindings and readings are, so they stay in the Senvia database only.

- Per-tenant access through existing Senvia authentication; the viewer never calls external services.

- All code and models owned by Sensovibe in your own repository; no dependency on claude.ai at run time.

# 12. Delivery plan

| Phase | Scope | Done when |
| --- | --- | --- |
| 1 Extract | Split today's library into packages/core; playground app works exactly as now | Bug-hunt results identical for all 94 models |
| 2 Export | CLI exports GLB + JSON for all 94 models with the schema in section 5 | All files under size budget, checks pass |
| 3 Viewer | packages/viewer with the API in section 6; wrapper for your front end | Loads any model; all modes work on a mid-range phone |
| 4 Backend | Tables in section 7; APIs for asset, points, bearings, status | Engineer can create an asset and map points |
| 5 Live link | WebSocket/MQTT point status and running speed into the viewer | Real sensor alarm colours a point and highlights the bearing |
| 6 Pilot | One customer site end to end | Site engineers use it for routine rounds |

Team: one 3D/R&D developer (library and export), one frontend developer (viewer), one backend developer (tables, APIs, live link), with you as domain reviewer for machine correctness and fault mapping.

# 13. Decisions needed from Sensovibe

- Front-end framework of Senvia (React, Angular, Vue, Flutter or plain web).

- Object storage choice (AWS S3, Azure Blob, self-hosted MinIO) and CDN.

- Live data channel already used by your gateways (MQTT broker or WebSocket service).

- Severity standard default (ISO 20816 zones) and any customer-specific limits.